CCNPv7.1 SWITCH

Chapter 3 Lab 3-1– Static VLANS, Trunking, and VTP

Topology

Objectives

Setup a VTP v2 Domain.

Create and maintain VLANs.

Configure 802.1Q Trunking.

Setup a VTP v3 Domain.

Background

VLANs logically segment a network by function, team, or application, regardless of the physical location of the users. End stations in a particular IP subnet are often associated with a specific VLAN. VLAN membership on a switch that is assigned manually for each interface is known as static VLAN membership.

Trunking, or connecting switches, and the VLAN Trunking Protocol (VTP) are technologies that support VLANs. VTP manages the addition, deletion, and renaming of VLANs on the entire network from a single switch.

Note: This lab was based on Cisco Catalyst 3650 running Cisco IOS cat3k_caa-universalk9.16.06.02.SPA.bin   and 2960 switches running Cisco IOS c2960-lanbasek9-mz.152-2.E7.bin.

Required Resources

2 Cisco 2960 with the Cisco IOS Release XE c2960-lanbasek9-mz.152-2.E7 or comparable

2 Cisco 3650 with the Cisco IOS XE Release c2960-lanbasek9-mz.152-2.E7 package installed or comparable

Home Computer with PuTTY software

Prepare for the Lab

Prepare the switches for the lab

Use the als_reset.tcl or  dls_reset.tcl script you created in Lab 1 “Preparing the Switch” to set your switches up for this lab. Then load the file BASE.CFG into the running-config with the command copy flash:BASE.CFG running-config. An example from DLS1:

DLS1# tclsh dls_reset.tcl

Erasing the nvram filesystem will remove all configuration files! Continue? [confirm]

[OK]

Erase of nvram: complete

Reloading the switch in 1 minute, type reload cancel to halt

Proceed with reload? [confirm]

*Mar  7 18:41:40.403: %SYS-7-NV_BLOCK_INIT: Initialized the geometry of nvram

*Mar  7 18:41:41.141: %SYS-5-RELOAD: Reload requested by console. Reload Reason: Reload command.

<switch reloads - output omitted>

Would you like to enter the initial configuration dialog? [yes/no]: n

Switch> en

*Mar  1 00:01:30.915: %LINK-5-CHANGED: Interface Vlan1, changed state to administratively down

Switch# copy BASE.CFG running-config

Destination filename [running-config]?

184 bytes copied in 0.310 secs (594 bytes/sec)

DLS1#

Configure basic switch parameters.

Configure an IP address on the management VLAN according to the diagram. VLAN 1 is the default management VLAN, but following best practice, we will use a different VLAN. In this case, VLAN 99.

Enter basic configuration commands on each switch according to the diagram.

Note: you are shown below an example for DLS1, you must put similar configuration on all switches

DLS1 example:

DLS1# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

DLS1(config)# interface vlan 99

DLS1(config-if)# ip address 10.1.99.101 255.255.255.0

DLS1(config-if)# no shutdown

The interface VLAN 99 will not come up immediately, because the broadcast domain it is associated with (VLAN 99) doesn’t exist on the switch. We will fix that in a few moments.

(Optional) On each switch, create an enable secret password and configure the VTY lines to allow remote access from other network devices.

DLS1 example:

DLS1(config)# enable secret class

DLS1(config)# line vty 0 15

DLS1(config-line)# password cisco

DLS1(config-line)# login

Note: The passwords configured here are required for NETLAB compatibility only and are NOT recommended for use in a live environment.

Note(2): For purely lab environment purposes, it is possible to configure the VTY lines so that they accept any Telnet connection immediately, without asking for a password, and place the user into the privileged EXEC mode directly. The configuration would be similar to the following example for DLS1: DLS1(config)# enable secret class DLS1(config)# line vty 0 15 DLS1(config-line)# no login DLS1(config-line)# privilege level 15

Configure VTP Version 2, VLANs, and Trunking.

A VTP domain, also called a VLAN management domain, consists of trunked switches that are under the same administrative responsibility sharing the same VTP domain name. A switch can be in only one VTP domain, and VLAN database contents in the domain are globally synchronized. VLAN information is not propagated until a domain name is specified and trunks are set up between the devices.

There are three versions of VTP available; Version 1 and 2 are able to support normal-range VLANs only, while version 3 can support normal- and extended-range VLANs, as well as the synchronization of other databases. Support for version 3 on the Catalyst platforms used in this lab was added in IOS version 12.2(52)SE. Older IOS versions do not generally support VTP version 3.

Switches operate in one of four VTP modes. The default VTP mode for the 2960 and 3650 switches is server mode, however our Lab 1 configuration changes this to transparent.

| VTP Mode | Description |
| --- | --- |
| VTP Server | You can create, modify, and delete VLANs and specify other configuration parameters, such as VTP version and VTP pruning, for the entire VTP domain. VTP servers advertise their VLAN configuration to other switches in the same VTP domain and synchronize their VLAN configuration with other switches based on advertisements received over trunk links. VTP server is the default mode. In VTP Server mode, VLAN configurations are only stored in the flash:vlan.dat file. While VLANs are manipulated in the configuration mode, the configuration commands do not appear in the running-config. |
| VTP Client | A VTP client behaves like a VTP server and transmits and receives VTP updates on its trunks, but you cannot create, change, or delete VLANs on a VTP client. VLANs are configured on another switch in the domain that is in server mode. In VTP Client mode, VLAN configurations are only stored in the flash:vlan.dat file. The configuration of VLANs does not appear in the running-config. |
| VTP Transparent | VTP transparent switches do not participate in VTP. A VTP transparent switch does not advertise its VLAN database nor synchronize its VLAN database based on received advertisements. However, transparent switches forward received VTP messages under two circumstances: either the VTP domain name of the transparent switch is empty (not yet configured), or it matches the domain name in the received VTP messages. In VTP Transparent mode, VLAN configurations are stored both in flash:vlan.dat file and also are present in the running-config. If extended range VLANs are used, however, they are stored in the flash:vlan.dat only if the switch is running VTP version 3. |
| VTP Off | A switch in VTP Off mode functions in the same manner as a VTP transparent switch, except that it does not forward VTP advertisements on trunks. VTP off is only available on switches that support VTP version 3 although it is not necessary to run VTP version 3 on the switch to be able to put it into the Off mode. In VTP Off mode, VLAN configurations are stored both in flash:vlan.dat file and also are present in the running-config. If extended range VLANs are used, however, they are stored in the flash:vlan.dat only if the switch is running VTP version 3. |

In this lab we will demonstrate the configuration and operation of both VTP versions 2 and 3. We will do this by first configuring VTP version 2 between DLS1 and ALS1, and then configuring DLS1, DLS2 and ALS2 with VTP version 3.

Topology

Verify VTP status

Issue the show vtp status command on DLS1

DLS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 1

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 64a0.e72a.2200

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Feature VLAN:

--------------

VTP Operating Mode                : Transparent

Maximum VLANs supported locally   : 1005

Number of existing VLANs          : 5

Configuration Revision            : 0

MD5 digest                        : 0x57 0xCD 0x40 0x65 0x63 0x59 0x47 0xBD

0x56 0x9D 0x4A 0x3E 0xA5 0x69 0x35 0xBC

Because no VLAN configurations were made, all settings except the VTP mode that was changed in Lab 1 are the defaults. This switch is capable of running version 1, 2 or 3 of VTP and runs version 1 by default. All switches in the VTP domain must run the same VTP version. The VTP mode is set to transparent as a result of steps performed in Lab 1. The number of existing VLANs is the five built-in VLANs. Different switches in the Catalyst family support different numbers of local VLANs. The 3560 switch used in this lab supports a maximum of 1,005 VLANs locally, while the 2960 switch used in this lab supports at most 255 VLANs. Lastly, note that the configuration revision is 0.

As you should recall from CCNA, the configuration revision number is compared amongst VTPv1 or VTPv2 switches and the VLAN database from the switch with the highest revision number is adopted by all the other switches in the VLAN management domain. Every time VLAN information is modified and saved in the VLAN database (vlan.dat), the revision number is increased by one when the user exits from VLAN configuration mode.

In VTPv3, revision numbers are still used but they no longer determine the switch whose database is going to apply to the entire domain. Instead, a single designated switch in a VTP domain called the primary server is allowed to assert its database in the entire VTP domain, even if its own revision number is lower. Other switches that are not primary servers are not allowed to assert their databases even if their revision numbers are higher.

Multiple switches in the VTP domain can be in VTP server mode. In VTPv1 and VTPv2, any of these server switches can be used to manage all other switches in the VTP domain. In VTPv3, a single primary server for a particular VTP domain is designated to control where changes originate from in the switched network. This enables careful management and protection of the VLAN database.

Configure VTP on DLS1.

We will start off this lab by configuring DLS1 for VTP Server mode and setting the VTP domain name and VTP version 2. We will also set a VTP password, which provides some rudimentary protection against automatic VLAN database propagation. Because this password is set, VTPv2 will not allow ALS1 to automatically learn the domain name once trunks are installed.

DLS1# conf t

Enter configuration commands, one per line.  End with CNTL/Z.

DLS1(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

DLS1(config)# vtp version 2

DLS1(config)# vtp mode server

Setting device to VTP Server mode for VLANS.

DLS1(config)# vtp password cisco123

Setting device VTP password to cisco123

DLS1(config)#

*Mar  1 00:29:10.895: %SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

Verify these settings by using the show vtp status command again.

DLS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 2

VTP Domain Name                 : SWLAB

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 64a0.e72a.2200

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Local updater ID is 0.0.0.0 (no valid interface found)

Feature VLAN:

--------------

VTP Operating Mode                : Server

Maximum VLANs supported locally   : 1005

Number of existing VLANs          : 5

Configuration Revision            : 0

MD5 digest                        : 0xEA 0x06 0x96 0x11 0xF2 0x29 0x99 0x4C

0xDC 0xFF 0xA4 0x86 0xE7 0x63 0xB7 0x81

Configure VLANs on DLS1

Next configure the VLANs that will be required to support the network. There are two ways to create VLANs, either directly via the vlan command or by assigning an interface to a non-existent VLAN. For now, you will create the VLANs directly on the switch. Create:

VLAN 99 to enable the management interface.

VLAN 999 as a “parking lot” VLAN for unused access ports

Suspend this VLAN to prevent ports in the VLAN from every communicating with each other.

The VLANs required for network operations, which are VLANs 100, 110, and 120.

Suspending a VLAN deserves a special mention. Each VLAN has an operational state associated with it: it can be either active (the default state) or suspended. A suspended VLAN exists but it does not operate. Access ports assigned to a suspended VLAN drop all frames and are unable to communicate, similar to ports put into a nonexistent VLAN. Putting a suspended VLAN back into the active state reinstates normal communication on ports in that VLAN.

To globally suspend a VLAN, use the state suspend command in the VLAN configuration mode. This state is propagated by VTP to all other switches in the VTP domain if VTP is in use.

To locally shut down a VLAN, use the shutdown command in the VLAN configuration mode. This setting is not propagated through VTP.

Do not confuse the shutdown command in the VLAN configuration mode with the same command available under interface Vlan mode, which has a different and unrelated meaning. Further discussion on suspending and reactivating VLANs can be found in Part 3, Step 7 of this lab.

DLS1(config)# vlan 99

DLS1(config-vlan)# name MANAGEMENT

DLS1(config-vlan)# vlan 100

DLS1(config-vlan)# name SERVERS

DLS1(config-vlan)# vlan 110

DLS1(config-vlan)# name GUEST

DLS1(config-vlan)# vlan 120

DLS1(config-vlan)# name OFFICE

DLS1(config-vlan)# vlan 999

DLS1(config-vlan)# name PARKING_LOT

DLS1(config-vlan)# state suspend

DLS1(config-vlan)# vlan 666

DLS1(config-vlan)# name NATIVE_DO_NOT_USE

DLS1(config-vlan)# exit

The VLANs will not appear in the VLAN database until the exit command is issued.

After configuring the VLANs, issue the show vtp status command and you will see that the all-important configuration revision number has increased based on these changes to the VLAN database. Note that the revision number you have when performing this lab may be different.

DLS1#show vtp status | include Configuration Revision

Configuration Revision            : 6

Configure trunking on DLS1

VTP will only propagate information over trunks. Cisco switches support Dynamic Trunking Protocol (DTP), which allows automatic negotiation of trunks. The partial output here from ALS1 shows you the default trunking mode:

ALS1#show interface f0/1 switchport

Name: Fa0/1

Switchport: Enabled

Administrative Mode: dynamic auto

Operational Mode: down

Administrative Trunking Encapsulation: dot1q

Negotiation of Trunking: On

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 1 (default)

Administrative Native VLAN tagging: enabled

Switches that are interconnected and have DTP enabled can form a trunk automatically if either end is in the dynamic desirable mode or static trunk mode on the condition that either both switches use the same VTP domain name or at least one of the switches does not yet have the VTP domain name configured.

The dynamic auto mode on both ends will prevent a trunk from automatically forming; however, this is not really a valid safeguard against unintentional trunk connections as the port can become a trunk if the other side changes to dynamic desirable or static trunk mode.

As a best practice you should configure each interface into either access or trunk mode and use the switchport nonegotiate interface configuration command to disable the propagation of DTP messages. Never leave ports to operate in the dynamic mode.

Configure the appropriate interfaces on DLS1 to be trunks.

Change the native VLAN from the default of VLAN 1 to VLAN 666.

Set the interfaces to be in trunking mode only, and include the switchport nonegotiate command.

The no shutdown command is needed because the Lab 1 configuration has all interfaces shutdown.

DLS1(config)# interface range g1/0/1-6

DLS1(config-if-range)# switchport trunk native vlan 666

DLS1(config-if-range)# switchport mode trunk

DLS1(config-if-range)# switchport nonegotiate

DLS1(config-if-range)# no shutdown

DLS1(config-if-range)#

By default, all VLANs are allowed on all trunks. You can explicitly control which VLANs are allowed on a trunk by using the switchport trunk allowed vlan vlan-id command on the interface at each end of the trunk.

There are several approaches to deciding what VLANs to allow or disallow to cross the trunk. Common practice is to disallow VLAN 1 and the PARKING_LOT vlan. You could go a step further and disallow any unused VLAN numbers, but you would then have to modify all the trunks should you later add a new VLAN to the network.

In this lab, disallowing the PARKING_LOT VLAN from all trunks is not really necessary since the VLAN has been suspended. Disallowing the VLAN can serve as an additional protection against inadvertent reactivation of this VLAN.

Disallowing VLAN 1, also referred to as VLAN 1 Minimization, excludes VLAN 1 from the trunk but does not restrict layer 2 management traffic (such as CDP, LLDP, VTP, STP, etc) from passing.

Since only these 2 VLANs are being disallowed, the except option of the command can be used:

DLS1(config-if-range)# switchport trunk allowed vlan ?

WORD    VLAN IDs of the allowed VLANs when this port is in trunking mode

add     add VLANs to the current list

all     all VLANs

except  all VLANs except the following

none    no VLANs

remove  remove VLANs from the current list

DLS1(config-if-range)# switchport trunk allowed vlan except 1,999

DLS1(config-if-range)#

Validate these settings by examining the switchport configuration for one of the trunk interfaces:

DLS1#show interface g1/0/1 switchport

Name: Gi1/0/1

Switchport: Enabled

Administrative Mode: trunk

Operational Mode: down    (note this is down as the other end of the link is shutdown)

Administrative Trunking Encapsulation: dot1q

Operational Trunking Encapsulation: dot1q

Negotiation of Trunking: Off

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 666 (NATIVE_DO_NOT_USE)

Administrative Native VLAN tagging: enabled

<output omitted>

Trunking VLANs Enabled: 2-998,1000-4094

<output omitted>

Configure VTP and trunking on ALS1

Next configure VTP and trunking on ALS1. Configure ALS1 to be in VTP Client mode and then configure all of the appropriate trunk interfaces to use a native VLAN of 666 and to be in trunking mode only. The native VLAN number does not have to be the same across your network, but it must match between switches on a given connected trunk. Also, disallow VLANs 1 and 999.

ALS1(config)# vtp mode client

Setting device to VTP Client mode for VLANS.

ALS1(config)# interface range f0/1-6

ALS1(config-if-range)# switchport trunk native vlan 666

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# switchport nonegotiate

ALS1(config-if-range)# switchport trunk allowed vlan except 1,999

ALS1(config-if-range)# no shutdown

ALS1(config-if-range)# exit

ALS1(config)#

After activating the interfaces, use the show interface trunk command to see the status of the trunks. You should see interfaces Fa0/1 and Fa0/2 in trunking mode.

ALS1# show interface trunk

Port        Mode             Encapsulation  Status        Native vlan

Fa0/1       on               802.1q         trunking      666

Fa0/2       on               802.1q         trunking      666

Port        Vlans allowed on trunk

Fa0/1       2-998,1000-4094

Fa0/2       2-998,1000-4094

Port        Vlans allowed and active in management domain

Fa0/1       none

Fa0/2       none

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       none

Fa0/2       none

ALS1#

Now if you look at the VTP status on ALS1, you will see the values are at their defaults, even though the trunk is operational. This is because of the VTP password.

ALS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 1

VTP Domain Name                 :

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 64a0.e72a.2200

Configuration last modified by 0.0.0.0 at 0-0-00 00:00:00

Feature VLAN:

--------------

VTP Operating Mode                : Client

Maximum VLANs supported locally   : 255

Number of existing VLANs          : 5

Configuration Revision            : 0

MD5 digest                        : 0x57 0xCD 0x40 0x65 0x63 0x59 0x47 0xBD

0x56 0x9D 0x4A 0x3E 0xA5 0x69 0x35 0xBC

Set the VTP password on ALS1 and the VLAN database will be synchronized. However, before you can set the password, the VTP domain name must be manually configured.

ALS1# conf t

Enter configuration commands, one per line.  End with CNTL/Z.

ALS1(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

ALS1(config)# vtp password cisco123

Setting device VTP password to cisco123

ALS1(config)# vtp version 2

Cannot modify version in VTP client mode unless the system is in VTP version 3

ALS1(config)# vtp version 3

ALS1(config)#

*Mar  1 01:52:01.833: %SW_VLAN-6-OLD_CONFIG_FILE_READ: Old version 2 VLAN configuration file detected and read OK.  Version 3

files will be written in the future.

ALS1(config)# vtp version 2

ALS1(config)# end

*Mar  1 00:27:21.902: %SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

Now check the VTP status and you will see a revision number matching that of DLS1, and that VLANs 99, 100, 110, 120, 666 and 999 are all in the local VLAN database.

ALS1# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 2

VTP Domain Name                 : SWLAB

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : 64a0.e72a.2200

Configuration last modified by 0.0.0.0 at 3-1-93 00:04:37

Feature VLAN:

--------------

VTP Operating Mode                : Client

Maximum VLANs supported locally   : 255

Number of existing VLANs          : 11

Configuration Revision            : 6

MD5 digest                        : 0x54 0xD0 0xBF 0xA5 0xBB 0x3F 0x15 0xB4

0x63 0x56 0xDC 0x06 0xAD 0x53 0xB6 0x22

ALS1# show vlan brief | incl active

1    default                          active    Fa0/1, Fa0/2, Fa0/3, Fa0/4

99   MANAGEMENT                       active

100  SERVERS                          active

110  GUEST                            active

120  OFFICE                           active

666  NATIVE_DO_NOT_USE                active

ALS1#

VLAN 999 will be missing from the filtered output above because it only includes VLANs in active state and VLAN 999 is suspended. Using the show vlan brief without filtering would show the VLAN 999.

You will also see that the configured VLANs except VLANs 1 and 999 are allowed over the trunks

ALS1#show interface trunk

Port        Mode             Encapsulation  Status        Native vlan

Fa0/1       on               802.1q         trunking      666

Fa0/2       on               802.1q         trunking      666

Port        Vlans allowed on trunk

Fa0/1       2-998,1000-4094

Fa0/2       2-998,1000-4094

Port        Vlans allowed and active in management domain

Fa0/1       99-100,110,120,666,999

Fa0/2       99-100,110,120,666,999

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       99-100,110,120,666

Fa0/2       99-100,110,120,666

ALS1#

You will also see that the state of interface VLAN 99 has changed to ‘up’.

*Mar  1 00:27:52.336: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan99, changed state to up

Because ALS1 is in VTP Client mode, local changes to the VLAN database cannot be made:

ALS1(config)# vlan 199

VTP VLAN configuration not allowed when device is in CLIENT mode.

ALS1(config)#

At this point, VTP version 2 is working and secured between DLS1 and ALS1. You should now be able to ping DLS1 from ALS1 and vice versa.

ALS1#ping 10.1.99.101

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.1.99.101, timeout is 2 seconds:

.!!!!

Success rate is 80 percent (4/5), round-trip min/avg/max = 1/3/9 ms

Park unused interfaces

On DLS1 and ALS1, place all of interfaces that will not be used into the PARKING_LOT VLAN and shut them down. For this lab, the interfaces being used on all switches are F0/1 through F0/6 or G1/0/1 through G1/0/6.

An example from DLS1:  you must do the equivalent for ALS1. The AL switches has links fa0/7-24 and g0/1-2.

DLS1(config)# interface range g1/0/7-24,g1/1/1-4

DLS1(config-if-range)# switchport mode access

DLS1(config-if-range)# switchport nonegotiate

DLS1(config-if-range)# switchport access vlan 999

DLS1(config-if-range)# shutdown

DLS1(config-if-range)# exit

Configure VTP version 3, VLANs, and Trunking

In this part of the lab you will configure VTP version 3 to operate across the rest of the switched network. VTP version 3 provides some significant benefits to the network administrator.

- The concept of a primary server was added. In VTP versions 1 and 2, all VTP server switches are equal; any one of them may add/remove/rename VLANs and change their state. In VTP version 3, only the primary server can do this. There can be at most one primary server present in a VTP domain. The role of a primary server is a runtime state. It is not a part of the configuration; rather, this state is requested in privileged EXEC mode and is relinquished whenever another switch attempts to become the primary server or when the switch is reloaded.

- VTP version 3 has the ability to hide the VTP password. On a VTP version 1 or 2 switch, issuing the command show vtp password will show the password to you in plain text. VTP version 3 allows you to specify that the password be hidden in the output, preventing the password from being inadvertently or maliciously divulged.

- VTP version 3 can propagate information about extended-range VLANs; VLANs numbered between 1006 and 4094. To support these VLANs with VTP version 2, all switches had to be in transparent mode and the VLANs had to be configured manually on a switch-by-switch basis.

- VTP version 3 only supports pruning for normal-range VLANs.

- VTP version 3 supports propagating Private VLAN information. As with extended-range VLANs, the lack of PVLAN support in VTP version 2 required all switches to be transparent mode and manual configuration at each switch.

- VTP version 3 added support for opaque databases. In other words, VTP version 3 can transport more than just the VLAN database between switches. The only option at this time is to share the Multiple Spanning Tree (MSTP) database, but room was left for expansion. We will cover MSTP in a later lab.

VTP version 3 is backwards compatible with VTP version 2; at the boundary of the two protocols, a VTP version 3 switch will send out both version 3 and version 2-compatible messages. Version 2 messages received by a version 3 switch are discarded.

Configure VTP on DLS1, DLS2, and ALS2

VTP version 3 cannot be configured unless a VTP domain name has been set, so for this step, setting the domain name is not needed on DLS1. Configure VTP version 3 on DLS1, DLS2, and ALS2 using the following parameters

VTP domain SWLAB (DLS2 and ALS2 only)

VTP version 3

VTP mode server (DLS2 and ALS2 only)

VTP password cisco123 (DLS2 and ALS2 only)

DLS1 Configuration:

DLS1(config)# vtp version 3

DLS1(config)#

*Mar  1 00:08:17.637: %SW_VLAN-6-OLD_CONFIG_FILE_READ: Old version 2 VLAN configuration file detected and read OK.  Version 3

files will be written in the future.

DLS1(config)# end

DLS1#

Example configuration on ALS2:

ALS2(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

ALS2(config)# vtp version 3

ALS2(config)# vtp mode server

Setting device to VTP Server mode for VLANS.

ALS2(config)# vtp password cisco123

Setting device VTP password to cisco123

ALS2(config)# end

*Mar  1 18:46:38.236: %SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

*Mar  1 18:46:38.345: %SW_VLAN-6-OLD_CONFIG_FILE_READ: Old version 2 VLAN configuration file detected and read OK.  Version 3

files will be written in the future.

ALS2#

Configure trunking on DLS2 and ALS2

In steps 4 and 5 of part 1, we configured and activated all the trunk interfaces on DLS1 and ALS1. Now configure and activate all the trunk interfaces on DLS2 and ALS2.

Example from DLS2:

DLS2(config)# interface range g1/0/1-6

DLS2(config-if-range)# switchport trunk native vlan 666

DLS2(config-if-range)# switchport mode trunk

DLS2(config-if-range)# switchport nonegotiate

DLS2(config-if-range)# switchport trunk allowed vlan except 1,999

DLS2(config-if-range)# no shutdown

DLS2(config-if-range)# exit

DLS2(config)#

Verify trunking and VTP on DLS2 and ALS2

Next verify that DLS2 and ALS2 trunking is operational. Here is an example from DLS2

DLS2# show interface trunk

Port          Mode             Encapsulation  Status        Native vlan

Gi1/0/1       on               802.1q         trunking      666

Gi1/0/2       on               802.1q         trunking      666

Gi1/0/3       on               802.1q         trunking      666

Gi1/0/4       on               802.1q         trunking      666

Gi1/0/5       on               802.1q         trunking      666

Gi1/0/6       on               802.1q         trunking      666

Port          Vlans allowed on trunk

Gi1/0/1       2-998,1000-4094

Gi1/0/2       2-998,1000-4094

Gi1/0/3       2-998,1000-4094

Gi1/0/4       2-998,1000-4094

Gi1/0/5       2-998,1000-4094

Gi1/0/6       2-998,1000-4094

Port          Vlans allowed and active in management domain

Gi1/0/1       none

Gi1/0/2       none

Gi1/0/3       none

Gi1/0/4       none

Gi1/0/5       none

Gi1/0/6       none

Port          Vlans in spanning tree forwarding state and not pruned

Gi1/0/1       none

Gi1/0/2       none

Gi1/0/3       none

Gi1/0/4       none

Gi1/0/5       none

Gi1/0/6       none

Next, validate VTP is operational. Here is an example from DLS2:

DLS2# show vtp status

VTP Version capable             : 1 to 3

VTP version running             : 3

VTP Domain Name                 : SWLAB

VTP Pruning Mode                : Disabled

VTP Traps Generation            : Disabled

Device ID                       : e840.406f.7380

Feature VLAN:

--------------

VTP Operating Mode                : Server

Number of existing VLANs          : 5

Number of existing extended VLANs : 0

Maximum VLANs supported locally   : 4096

Configuration Revision            : 0

Primary ID                        : 0000.0000.0000

Primary Description               :

MD5 digest                        :

Feature MST:

--------------

VTP Operating Mode                : Transparent

Feature UNKNOWN:

--------------

VTP Operating Mode                : Transparent

DLS2#

Notice that the configuration revision number is zero and the number of local VLANs is the default of 5. There has been no update because DLS1’s configuration revision number was reset to zero when the VTP version was changed, so at this point DLS2 and ALS2 will not learn about the configured VLANs because as far as they are concerned, they have the same database as DLS1.

DLS1# show vtp status | inc Configuration Revision

Configuration Revision            : 0

If we attempt to add VLANs at DLS1, or any of the other VTP version 3 switches, our attempt will not work and we will be told that we cannot add VLANs.

DLS1(config)# vlan 111

VTP VLAN configuration not allowed when device is not the primary server for vlan database.

DLS1(config)#

Configure the Primary VTP Server

In a VTP version 3 domain, only the “Primary Server” can make changes to the VLAN database. Becoming the primary server requires the vtp primary privileged EXEC command be executed. When you issue that command, the switch checks to see if there is another switch acting as primary server already, and asks you to confirm that you want to continue.

In the output from DLS2 above, note that the Primary ID field equals 0000.0000.0000. That field will display the base MAC address of the primary server once a device is promoted into that role.

Also note that a separate primary server can be configured independently for each feature supported; VLAN or MST. If no feature is specified, the vlan feature is assumed.

Lastly, there is a force option which causes the switch not to check for conflicts in the identity of the primary server. If different switches in the VTP domain identify different switches as the primary server, there is a good chance there are inconsistencies in the VLAN database.

DLS1# vtp primary ?

force  Do not check for conflicting devices

mst    MST feature

vlan   Vlan feature

<cr>

DLS1# vtp primary vlan

This system is becoming primary server for feature vlan

No conflicting VTP3 devices found.

Do you want to continue? [confirm]

DLS1#

*Mar  1 00:42:54.983: %SW_VLAN-4-VTP_PRIMARY_SERVER_CHG: e840.406f.7280 has become the primary server for the VLAN VTP feature

Now verify the primary on DLS2 or ALS2:

DLS2# show vtp status | i Primary

Primary ID                        : e840.406f.7280

Primary Description               : DLS1

DLS2#

The promotion of DLS1 to primary increments its configuration revision number to 1, so the VLANs that were previously created on DLS1 are propagated to DLS2 and ALS2 automatically.

ALS2# sho vlan brief | inc active

1    default                          active    Fa0/7, Fa0/8, Fa0/9, Fa0/10

99   MANAGEMENT                       active

100  SERVERS                          active

110  GUEST                            active

120  OFFICE                           active

666  NATIVE_DO_NOT_USE                active

VLAN 999 will be missing from the filtered output above because it only includes VLANs in active state and VLAN 999 is suspended. Using the show vlan brief without filtering would show the VLAN 999.

Park unused interfaces.

On DLS2 and ALS2, place all of interfaces that will not be used into the PARKING_LOT VLAN and shut them down. For this lab, the interfaces being used on all switches are F0/1 through F0/6 or G1/0/1 through G1/0/6.

An example from DLS2:  you must do the equivalent for ALS2. The AL switches has links fa0/7-24 and g0/1-2.

An example from DLS2:

DLS2(config)# interface range g1/0/7-24,g1/1/1-4

DLS2(config-if-range)# switchport mode access

DLS2(config-if-range)# switchport nonegotiate

DLS2(config-if-range)# switchport access vlan 999

DLS2(config-if-range)# shutdown

DLS2(config-if-range)# exit

Verify VLAN management capability

DLS1 is able to create VLANs, including extended-range VLANs. Note that because ALS1 is running VTP version 2 and its revision number is 6, it will ignore any of the VTPv2 messages sent to it because they have a lower revision number. When a VTP message with an equal revision number but different MD5 checksum is received, ALS1 will report an error. Here we added seven VLANs and then remove 6 of them to push the revision number on DLS1 to 9.

DLS1# conf t

Enter configuration commands, one per line.  End with CNTL/Z.

DLS1(config)# vlan 510

DLS1(config-vlan)# name TEST510

DLS1(config-vlan)# exit

NOTE: you will probably see a message like this on all devices

SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 511 on GigabitEthernet1/0/11 VLAN666.

This is because the DLS switches are defaulting to Rapid per vlan spanning tree and the ALS switches are IEEE per vlan spanning tree

It is fine to receive these messages, but to make Spanning tree consistent on DLS1 and DLS2 you can change the spanning tree mode by entering:

DLS1(config)# spanning-tree mode pvst

DLS1(config)# vlan 511

DLS1(config-vlan)# name TEST511

DLS1(config-vlan)# exit

DLS1(config)# vlan 512

DLS1(config-vlan)# name TEST512

DLS1(config-vlan)# exit

DLS1(config)# vlan 513

DLS1(config-vlan)# name TEST513

DLS1(config-vlan)# exit

DLS1(config)# vlan 514

DLS1(config-vlan)# name TEST514

DLS1(config-vlan)# exit

DLS1(config)# vlan 515

DLS1(config-vlan)# name TEST515

DLS1(config-vlan)# exit

DLS1(config)# vlan 1500

DLS1(config-vlan)# name TEST-EXT-1500

DLS1(config-vlan)# exit

DLS1(config)# no vlan 510-514

DLS1(config)# end

DLS1#

ALS1# show vlan brief | i active

1    default                          active

99   MANAGEMENT                       active

100  SERVERS                          active

110  GUEST                            active

120  OFFICE                           active

515  TEST515                          active

666  NATIVE_DO_NOT_USE                active

ALS1#

ALS2#show vlan brief | inc active

1    default                          active

99   MANAGEMENT                       active

100  SERVERS                          active

110  GUEST                            active

120  OFFICE                           active

515  TEST515                          active

666  NATIVE_DO_NOT_USE                active

1500 TEST-EXT-1500                    active

ALS2#

You should notice that the extended VLAN 1500 is not able to be conveyed to ALS1 as ALS1 is using vtp ver 2 which does not support extended vlans.

At this point, you should be able to ping each switch from every other switch.

Change the VLAN status to deactivate ports.

As already briefly discussed in Part 2, Step 3 of this lab, the default status of VLAN 1 and user-created VLANs is "active". A VLAN can be made locally inactive by entering the global configuration command shutdown vlan vlan-id, where vlan-id is the number of the VLAN to be shut down.

Alternatively, the VLAN can be shut down by issuing the shutdown command while in VLAN configuration mode and then exiting.

Both these options are equivalent, however, only the shutdown vlan command works in while in VTP client mode.

Shut down the Guest VLAN 110 on ALS1, wait a few moments, exit the configuration mode and then issue the show vlan brief command. The status should change to “act/lshut”.

ALS1(config)# shutdown vlan 110

ALS1# show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active

99   MANAGEMENT                       active

100  SERVERS                          active

110  GUEST                            act/lshut

120  OFFICE                           active

515  TEST515                          active

666  NATIVE_DO_NOT_USE                active

999  PARKING_LOT                      suspended Fa0/7, Fa0/8, Fa0/9, Fa0/10

Fa0/11, Fa0/12, Fa0/13, Fa0/14

Fa0/15, Fa0/16, Fa0/17, Fa0/18

Fa0/19, Fa0/20, Fa0/21, Fa0/22

Fa0/23, Fa0/24, Gi0/1, Gi0/2

1002 fddi-default                     act/unsup

1003 trcrf-default                    act/unsup

1004 fddinet-default                  act/unsup

1005 trbrf-default                    act/unsup

ALS1#

Reactivate all ports in ALS1 Guest VLAN 110 using the no shutdown command in VLAN configuration mode.

ALS1(config)# no shutdown vlan 110

As discussed and demonstrated in Part 2, Step 3 of this lab, you can put a VLAN into suspended status by using the state suspend command while in VLAN configuration mode on a VTPv2 server switch or on the VTPv3 primary server switch. In a mixed VTP version network, the suspension only works network-wide if it originates from the VTPv3 primary server. Suspending a VLAN causes all ports in that VLAN throughout the VTP domain to stop transferring data.

Suspend Guest VLAN 110 on DLS1, wait a few moments, exit VLAN configuration mode and then issue the show vlan brief | include suspended command. The status should change show the VLAN as suspended.

DLS1(config)# vlan 110

DLS1(config-vlan)# state ?

active   VLAN Active State

suspend  VLAN Suspended State

DLS1(config-vlan)# state suspend

DLS1(config-vlan)# exit

DLS1(config)#end

DLS1#

DLS1# sho vlan brief | include suspended

110  GUEST                            suspended

999  PARKING_LOT                      suspended Gi1/0/7, Gi1/0/8, Gi1/0/9, Gi1/0/10

DLS1#

Issue the show vlan brief | include suspended command on another switch in the network, and you will see that the VLAN status is suspended as well.

DLS2# show vlan brief | include suspended

110  GUEST                            suspended

999  PARKING_LOT                      suspended Gi1/0/7, Gi1/0/8, Gi1/0/9, Gi1/0/10

DLS2#

Reactivate VLAN 110 using the state active command in VLAN configuration mode.

DLS1(config)# vlan 110

DLS1(config-vlan)# state active

DLS1(config-vlan)# exit

DLS1(config)#

Issue the show vlan brief | include suspended command on another switch in the network, and you will see that the VLAN status is no longer listed.

DLS2# show vlan brief | include suspended

999  PARKING_LOT                      suspended Gi1/0/7, Gi1/0/8, Gi1/0/9, Gi1/0/10

DLS2#

End of Lab

Save your configurations. The equipment should be in the correct end state from this lab for Lab 3-2, EtherChannel.